Recap

Giving this another try. Starting with the simpler constraint given in the paper,

NQLLNtot=N-N*sin⁡(2πNtot) 							(1)

The idea is to begin with the requirement that

NQLLo+dNQLLdt∆t'=NQLL(Ntoto+dNtotdt∆t')						(2)

where dNtotdt depends on the usual diffusion and source terms, and the right-hand-side is the function specified in Eq. 1 but with Ntoto+dNtotdt∆t' replacing Ntot. The value of ∆t' is arbitrary, since the intention is that it will go away by the time we’re done. If we solve the above equation for dNQLLdt and take a Taylor expansion of Eq. 1 about NQLLo, we find that ∆t' really does cancel out, giving

dNQLL,Taylordt≈-N*×2π cos⁡(2πNtoto)×dNtotdt 					(3)

where ≈ is meant to remind us that this is the result of a Taylor expansion. By contrast, the original algorithm (in the paper) is very similar to this,

dNQLL,Oridt=-N*×2π cos⁡(2πNtoto)×dNtotdt + D∇2NQLL				(4)

The “source” term in Eq. 3 is identical to the one in Eq. 4 (omitting a factor of N that I think shouldn’t be there -- in any case, N=1 in our simulations, so in practice it doesn’t make a difference). The big difference between Eqs. 3 and 4 is D∇2NQLL.

| Figure 1. Steady state using the original formulation (i.e., Eq. 4), with D=2×10-7 μm2μs. |  |
| --- | --- |

In numerical simulations, I’ve found that D controls whether the system achieves steady-state: the smaller D, the more steps in the steady state. Fig. 1 shows the results when D=2×10-7μm2μs, which is 1/1000 the value used in the paper’s “reference state.” This run exhibited a diffusive slowdown of about 2%.

Figure 2 shows results with the biggest value of D I can specify and still get converged results without interrupting to update NQLL (again assuming other parameters assume their “reference” values), D=2×10-6μm2μs, which is 1/100 the value used in the paper’s “reference state.” Here, the simulation runs fast enough that it’s easy to go well past the first point of “steady state” (here, at 30,000 μs)  shown. What is revealed here, is that the steady state is not actually steady: the profile exhibits an inversion. This run also exhibited a diffusive slowdown of about 2%.

Figure 2. Layer difference (left), and profiles of states close to 30,000 μs (middle) and the ending state (right), using the original formulation for dNQLL,SSEdt (i.e., Eq. 4), and D as shown.

If we make the size of the crystal bigger, we can get to bigger diffusion coefficients. Figure 3 shows a run having a crystal twice as long as the previous, and D=1×10-5μm2μs, which is 1/20 the value used in the paper’s “reference state.”

Figure 3. Layer difference (left), and profiles of states close to 30,000 μs (right), using the original formulation for dNQLL,SSEdt (i.e., Eq. 4), D as shown, but a bigger crystal.

Focusing on 260 K

Now we’re going to see if we can be more specific about the temperature, say T=260 K. According to Gladich et al’s, Table 2 (reproduced below), at that temperature we should have D=0.16×10-9m2s=1.6×10-4μm2μs.

From Gladich et al paper:

We also need the kinetic deposition rate. The formula is given by Neshyba et al’s Eq. 2, which will be our Eq. 5 here:

νkin=csatcsolkT2πm 									(5)

where csolis the density of ice, csat is the density of water vapor in equilibrium with the solid, and m is the mass of a water molecule. Using this, we get νkin=250μms. That’s a lot bigger than what is used in the paper, 49μms, but that value would apply to a lower temperature, probably T=250 K.

Bearing in mind that computational stability requires that source term in Eq. 4 needs to be big compared to the diffusion term, D∆x2 (variable Doverdeltax2 in the code), we can note the following:

- The source term, -N*×2π cos⁡(2πNtoto)×dNtotdt (which remember we want to be big), increases with bigger νkin, so it’s good that we’re up to νkin=250μms now. The supersaturation, designated in the paper as σI,corner (see Eq. 13 of the paper, and appearing in the old code as variable sigmastepmax, revised to sigmaIcorner), had a value of σI,corner=0.20 for the reference system in the paper. If we bump this up to σI,corner=0.22, it buys us some maneuvering room. As long as σI,corner>σo=0.19, we’ll get growth.

- In the diffusion term, the key quantity is D∆x2 (which remember we want to be small). We’re stuck with D=1.6×10-4μm2μs if we want to adhere to what Gladich et say for T=260 K in their Table 2. We do have some control over ∆x , however: it’s determined by how we discretize. If we choose a bigger crystal with the same number of points, ∆x is bigger and therefore D∆x2 is smaller, so that’s good. There’s a limit to this, however, because if ∆x gets too small, we lose the ability to resolve the steps. It seems like ∆x needs to be no greater than 0.3 μm to achieve this.

Figure 4 shows a run having a 150 μm crystal (so L=75 μm), with parameters corresponding to a temperature of T=260 K. It achieves what we’ve been calling a steady state, although now we recognize that it inverts.

Figure 4. Layer difference (left), and profile close to 25,000 μs (right), with νkin=250μms, σI,corner=0.22, and D as shown. For this discretization, ∆x=0.3 μm.

My lazy way of doing Clausius-Clapeyron

(https://byjus.com/clausius-clapeyron-equation-calculator/)

Figure 5 shows results of another run with smaller D, just to make sure that the inversion also happens with strong V-shaped profiles (it does). What’s also striking to me is the way NQLL has a different behavior around the times that the surface is flat (layer difference close to zero).

Figure 5. Layer difference (left), profile close to 10,000 μs (center), and NQLL, with νkin=250μms, σI,corner=0.22, and D as shown.